Name: Utkarsh Pandya

Module -2

Section 1: Multiple Choice

1. Which of the following precautions should be taken before working on computer hardware?

a) Ensure the computer is plugged in to prevent electrostatic discharge.

b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

c) Work on carpeted surfaces to prevent slipping.

d) Use magnetic tools to handle components more easily

2.What is the purpose of thermal paste during CPU installation?

- To insulate the CPU from heat.

- To provide mechanical support for the CPU.

- To improve thermal conductivity between the CPU and the heat sink.

- To prevent the CPU from overheating.

3. Which tool is used to measure the output voltage of a power supply unit (PSU)?

a) Multimeter

b) Screwdriver

c) Pliers

d) Hex key

4. Which component is responsible for storing BIOS settings, such as date and time, even when the computer is powered off?

a) CMOS battery

b) CPU

c) RAM

d) Hard drive

Section 2: True or False

5. True or False: When installing a new hard drive, it is essential to format it before use. - True

6. True or False: A POST (Power-On Self-Test) error indicates a problem with the CPU. - False

7. True or False: It is safe to remove a USB flash drive from a computer without ejecting it first. – False

Section 3: Short Answer

8. Describe the steps involved in installing a new graphics card in a desktop computer.

Ans :

- Power Off and Unplug

  - Shut down the computer and disconnect the power cable and all peripherals.

- Open the Computer Case

  - Remove the side panel using a screwdriver to access the motherboard.

- Remove the Old Graphics Card (if installed)

  - Unscrew the bracket.

  - Unlock the PCIe slot latch.

  - Gently pull out the old GPU.

- Prepare the New Graphics Card

  - Remove protective covers from the card’s connectors.

  - Align the card with the PCIe x16 slot.

- Insert the New Graphics Card

  - Firmly press the card into the PCIe slot until it clicks.

  - Ensure the PCIe latch locks in place.

- Secure the Card

  - Screw the GPU bracket to the case to hold it firmly.

- Connect Power Cables

  - Attach PCIe power connectors (6-pin / 8-pin) from the power supply if required.

- Close the Case

  - Reattach the side panel of the cabinet.

- Reconnect and Power On

  - Plug in the power cable and connect the monitor to the new GPU.

- Install Drivers

- Boot the computer.

- Download and install the latest drivers from NVIDIA/AMD’s website

9. What is RAID, and what are some common RAID configurations?

Ans : RAID (Redundant Array of Independent Disks) is a technology that combines multiple hard drives into a single logical unit to improve performance, data redundancy, or both. RAID helps protect data against disk failures and enhances storage efficiency.

Common RAID Configurations

1. RAID 0 (Striping)

- Data is split across multiple disks.

- Advantages: High performance and speed.

- Disadvantages: No redundancy; if one disk fails, all data is lost.

2. RAID 1 (Mirroring)

- Data is duplicated on two or more disks.

- Advantages: High data reliability and fault tolerance.

- Disadvantages: Requires double storage; costly.

3. RAID 5 (Striping with Parity)

- Data and parity information are distributed across three or more disks.

- Advantages: Good performance and fault tolerance; efficient storage use.

- Disadvantages: Rebuild time after a disk failure can be slow.

4. RAID 10 (Combination of RAID 1 + RAID 0)

- Data is striped and mirrored.

- Advantages: High performance + strong redundancy.

- Disadvantages: Requires at least 4 disks; expensive.

Section 4: Practical Applicatio

10. Demonstrate how to replace a CPU fan in a desktop computer

ANS= 1. Power Down and Unplug

• 	Shut down the computer.

• 	Unplug the power cable and all peripherals.

• 	Press the power button once (while unplugged) to discharge any residual electricity.

2. Open the Case

• 	Use a screwdriver to remove the side panel of the CPU cabinet.

• 	Place the case on a clean, flat surface.

3. Ground Yourself

• 	Wear an anti-static wrist strap or touch a metal part of the case to discharge static electricity.

4. Locate the CPU Fan

• 	The CPU fan is mounted on top of the CPU, usually attached to a heat sink.

• 	It’s connected to the motherboard via a small 3-pin or 4-pin connector.

5. Disconnect the Fan Cable

• 	Gently unplug the fan’s power cable from the motherboard header labeled CPU_FAN.

6. Remove the Old Fan and Heat Sink

• 	Depending on the mounting system (clips, screws, or push pins), carefully release or unscrew the fan and heat sink.

• 	Lift it off gently — don’t force it.

7. Clean the CPU Surface

• 	Use isopropyl alcohol and a lint-free cloth to clean off old thermal paste from the CPU surface.

8. Apply New Thermal Paste

• 	Apply a small pea-sized drop of thermal paste to the center of the CPU.

• 	This ensures proper heat transfer between the CPU and the new cooler.

9. Install the New CPU Fan

• 	Align the new fan and heat sink over the CPU.

• 	Secure it using the appropriate mounting method (clips, screws, or push pins).

• 	Plug the fan’s power cable into the CPU_FAN header on the motherboard.

10. Close the Case and Reconnect

• 	Reattach the side panel.

• 	Plug in the power cable and peripherals.

• 	Boot up the system and check BIOS or software to confirm the fan is running properly.

Section 5: Essay

11. Discuss the importance of regular maintenance for computer hardware and provide examples of maintenance tasks.

Ans :

Regular maintenance ensures that computer hardware remains reliable, efficient, and long-lasting. It helps prevent unexpected failures, improves performance, and reduces repair costs. Maintenance also ensures that components stay clean, cool, and free from physical or electrical issues.

Why Regular Maintenance Is Important

- Prevents overheating by removing dust and improving airflow.

- Increases lifespan of components like CPU, GPU, fans, and storage.

- Improves system performance by keeping hardware running smoothly.

- Reduces risk of hardware failure and data loss.

- Ensures stable operation for both personal and professional use.

Examples of Computer Hardware Maintenance Tasks

1. Cleaning and Dust Removal

- Use compressed air to clean fans, vents, heat sinks, and the motherboard.

- Prevents overheating and fan noise.

2. Checking and Tightening Connections

- Ensure cables, RAM, and expansion cards are properly seated.

- Avoids intermittent hardware issues.

3. Updating Drivers and Firmware

- Improves compatibility, performance, and security.

4. Hard Disk Maintenance

- Run disk cleanup.

- Defragment HDDs (not needed for SSDs).

- Check disk health using tools like CHKDSK.

5. Monitoring System Temperatures

- Use software to track CPU/GPU temperature.

- Helps detect cooling issues early.

6. Replacing Thermal Paste

- Keeps CPU and GPU temperatures low.

- Recommended every 1–2 years.

7. Power Supply and Battery Check

- Ensure stable power delivery.

- Replace faulty or old power supply units.

8. Cleaning Peripherals

- Keyboard, mouse, monitor, and ports to maintain good usability.